Components table

| Category | Component Name | Qty | Unit Price | Total Cost |
| --- | --- | --- | --- | --- |
| Actuators | MG996R 180° Servos (torque) (9.4 kgf·cm 4.8V) | 3 | Pack of three motors | ₹1,317 |
| Actuators | MG90S (metal-gear) Servos(2.2 kg·cm ) | 3 | ₹600 | ₹439 |
| Main Brain | ESP32 Development Board | 1 | ₹342 | ₹342 |
|  | GPIO expansion board | 1 | ₹120 | ₹120 |
| Vision Input | # ESP32 CAM Development Board | 1 | ₹229 | ₹229 |
|  | ESP32-CAM (with OV2640) | 1 | ₹849 | ₹849 |
| Programmer | FTDI Programmer (USB-to-TTL) | 1 | ₹150 | ₹150 |
| Servo Driver | PCA9685 16-Channel Servo Driver | 1 | ₹249 | ₹249 |
| Power Converter | XL4015 5A Buck Converter | 1 | ₹120 | ₹120 |
| Power Source | 3S LiPo Battery 22mah | 1 | ₹1,899 | ₹1,899 |
|  | # Robocraze iMAX B6 Digital Balance Charger | 1 | ₹2,689 | ₹2,689 |
| For extending the servo wires | # 30CM Servo Extension Cable Pack | 1 | ₹299 | ₹299 |
| Wiring & Base | Male-female jumper wires | 1 Pack | ₹200 | ₹200 |

Total Project Cost:- Estimated Total Hardware Cost: Rs 8,514.

Components  we are  going to  use :-

MG90S Metal Gear Micro Servo Motor

Lipo battery  imax  charger

Servo Extension  cables  (30cm)

ESP32 Development Board

Lipo  2200mah  battery

MG996R Metal Gear  Servo Motor

ESP32 Cam  Module

ESP32  GPIO Extension  board

ESP32 Cam  Module Extension  board

FTDI Programmer (USB-to-TTL)

XL4015 5A Buck Converter

PCA9685 16-Channel Servo Driver

### Specifications to Look For on the Battery Label

When browsing the options in your college lab, pick a pack that matches these specs:

- Cell Count: 3S or 11.1V

- Capacity: 2200mAh to 3300mAh (provides 30–45 minutes of active movement testing)

Power Supply & Voltage Regulation Wiring

| From Source | Pin / Terminal | To Destination | Pin / Terminal | Purpose |
| --- | --- | --- | --- | --- |
| 3S LiPo Battery | Red Wire (+) | XL4015 Buck Converter | IN+ | Raw 11.1V Battery Input |
| 3S LiPo Battery | Black Wire (-) | XL4015 Buck Converter | IN- | Battery Ground |
| XL4015 Output | OUT+ (Set to 6.0V) | PCA9685 Driver | Green Screw Terminal V+ | Servo Power Rail |
| XL4015 Output | OUT- (GND) | PCA9685 Driver | Green Screw Terminal GND | Motor Ground |
| XL4015 Output | OUT+ (Set to 6.0V) | ESP32 Expansion Board | VIN (or 5V) | Board Main Power |
| XL4015 Output | OUT- (GND) | ESP32 Expansion Board | GND | Shared System Ground |

Control & I2C Signal Wiring (ESP32 to PCA9685)

| From Source (ESP32 Expansion Board) | Pin / Terminal | To Destination (PCA9685 Driver) | Pin / Terminal | Purpose |
| --- | --- | --- | --- | --- |
| ESP32 Expansion Board | 3.3V | PCA9685 Driver | VCC | Logic Power for Driver IC |
| ESP32 Expansion Board | GND | PCA9685 Driver | GND | Shared Signal Ground |
| ESP32 Expansion Board | GPIO 21 | PCA9685 Driver | SDA | I2C Data Line |
| ESP32 Expansion Board | GPIO 22 | PCA9685 Driver | SCL | I2C Clock Line |

PCA9685 Updated Driver Output Wiring

| PCA9685 Channel | Wire Alignment | Connected Servo | Joint Control |
| --- | --- | --- | --- |
| Channel 0 | Yellow/Orange (PWM), Red (V+), Brown/Black (GND) | MG996R | Base Rotation |
| Channel 1 | Yellow/Orange (PWM), Red (V+), Brown/Black (GND) | MG996R | Shoulder Lift |
| Channel 2 | Yellow/Orange (PWM), Red (V+), Brown/Black (GND) | MG996R | Elbow Movement |
| Channel 3 | Yellow/Orange (PWM), Red (V+), Brown/Black (GND) | MG90S | Wrist Pitch |
| Channel 4 | Yellow/Orange (PWM), Red (V+), Brown/Black (GND) | MG90S | Gripper Actuation |
| Channel 5 | Yellow/Orange (PWM), Red (V+), Brown/Black (GND) | MG90S | Camera Mount / Spare Joint |