1. Dimension Master

Definition: A single source of truth containing every dimension that controls your robotic arm.

Mechanism: Instead of randomly measuring things while modelling, we establish the important dimensions first and use those dimensions throughout Fusion.

Real-World Use-Case: If we later discover that your servo mounting-hole spacing is different, we change one parameter and the related bracket/link geometry can be updated.

I want us to eventually have a table like:

| Component | Dimension | Value | Source |
| --- | --- | --- | --- |
| MG996R | Body length | TBD | Physical measurement |
| MG996R | Body width | TBD | Physical measurement |
| MG996R | Body height | TBD | Physical measurement |
| MG996R | Mounting-hole spacing | TBD | Physical measurement |
| MG996R | Mounting-hole diameter | TBD | Physical measurement |
| MG996R | Output shaft height | TBD | Physical measurement |
| MG996R | Spline type | TBD/verified | Servo specification |
| Horn | Outer diameter | TBD | Physical/CAD measurement |
| Horn | Thickness | TBD | Physical/CAD measurement |
| Horn | Hole-circle diameter | TBD | Physical/CAD measurement |
| Horn | Hole diameter | TBD | Physical/CAD measurement |
| Link 1 | Length | TBD | Our design |
| Link 1 | Width | TBD | Our design |
| Link 1 | Thickness | TBD | Our design |
| Link 2 | Length | TBD | Our design |
| ... | ... | ... | ... |

TBD is intentional.

I don't want to give you a number just because it appears on some random MG996R webpage.

2. The MG996R becomes our mechanical reference

This is probably the most important component in the entire arm.

Your arm isn't going to be:

Link → servo somehow

It should be:

SERVO

│

┌──────────┴──────────┐

│                     │

Servo body            Output shaft

│                     │

mounting                  horn

geometry                  │

│                     │

└──────────┬──────────┘

│

ARM LINK

Every mounting feature on our printed parts should be designed around the actual servo geometry.

3. We need to distinguish THREE kinds of dimensions

This is very important.

A. Manufacturer dimensions

These come from reliable specifications/datasheets.

Example:

Servo body dimensions

Spline specification

Operating angle

Torque

B. Physical measurements

These are dimensions we should measure from the actual servo/horn you will use.

For example:

Mounting-hole diameter

Hole-to-hole distance

Horn thickness

Shaft protrusion

Actual body dimensions

C. Our design dimensions

These are things we decide:

Link length

Link thickness

Bracket thickness

Fillet radius

Wall thickness

Bearing clearance

We should never confuse these three.

4. Physical measurement is going to be VERY important

This is the part I don't want us to skip.

If you have the actual MG996R servo with you, the best tool would be a:

Digital Vernier Caliper

Something like:

Digital Vernier Caliper

┌────────────────────────┐

│      25.43 mm          │

└────────────────────────┘

\              /

\            /

────────────

With it, we can measure the physical servo to around 0.01 mm display resolution, although the practical accuracy depends on the caliper, technique, and surface.

If you don't have a caliper, we can still start with CAD/reference dimensions, but before printing the final parts I strongly recommend measuring the actual hardware.

5. Servo Horn

We've now got this beautiful circular horn in Fusion:

○   ○   ○

○               ○

○        ◎          ○

○               ○

○   ○   ○

But before attaching it permanently to the servo model, we'll verify:

Center spline

Does the horn actually fit the MG996R's output spline?

Outer diameter

How far does the horn extend?

Thickness

How thick should our mating component be?

Hole pattern

This is extremely important because your arm link may attach through those holes.

Hole diameter

We need to know what screw will actually pass through them.

6. Mechanical clearance

This is where 3D printing becomes different from simply modelling.

Suppose we model:

Screw = 3.00 mm

Hole = 3.00 mm

That doesn't necessarily mean the screw will comfortably pass through a printed hole.

We may instead need something like:

Nominal screw

↓

3.00 mm

↓

Printed clearance hole

↓

slightly larger

The exact clearance depends on:

- printer

- nozzle

- material

- layer height

- print orientation

- whether the hole is intended to be loose or tapped

So we will distinguish:

NOMINAL DIMENSION

↓

MANUFACTURING CLEARANCE

↓

FINAL PRINTED DIMENSION

7. Wall thickness

For your arm links, we shouldn't simply make a thin decorative shell.

We'll think about:

Link

┌─────────────────┐

│                 │

│    internal     │

│    structure    │

│                 │

└─────────────────┘

We'll consider:

- load

- servo torque

- bending

- weight

- printability

- infill

- layer orientation

- screw mounting

- stress concentration

Because your MG996R can produce significant torque, the joint areas deserve special attention.

8. Joint design

This is where the arm becomes an actual robot.

For example:

LINK 2

│

│

┌────┴────┐

│  JOINT  │

│ MG996R  │

└────┬────┘

│

LINK 1

We'll determine:

- rotation axis

- servo position

- horn position

- link centerline

- mounting brackets

- screw locations

- mechanical stops

- collision envelope

9. We also need to calculate the arm before printing

This is something I particularly want us to do.

Suppose eventually we have:

Base

│

├──── Link 1 = L1

│

├──── Link 2 = L2

│

└──── Link 3 = L3

We'll calculate the arm's:

Reach

How far can the end effector reach?

Payload

How much weight can the arm realistically handle?

Torque

How much torque does each joint need?

Center of mass

Where does the weight of the links and payload act?

Servo loading

This is critical because a servo may have enough torque at one joint but become overloaded at another.

10. Then we design the actual Fusion model

Our Fusion project can eventually become:

ROBOTIC ARM

│

├── MASTER PARAMETERS

│

├── BASE

│

├── WAIST

│

├── JOINT 1

│   ├── MG996R

│   ├── Horn

│   └── Mount

│

├── LINK 1

│

├── JOINT 2

│   ├── MG996R

│   ├── Horn

│   └── Mount

│

├── LINK 2

│

├── JOINT 3

│

├── LINK 3

│

└── END EFFECTOR

And importantly, we'll make the printed parts separate components.

That makes:

- editing easier

- exporting STL easier

- assembly easier

- printing easier

- replacement easier

🖨️ 11. Before we print anything

We will do a complete Print Readiness Check.

☐ Dimensions verified

☐ Servo mounting verified

☐ Horn fit verified

☐ Screw holes verified

☐ Clearances verified

☐ Joint axes verified

☐ No component collisions

☐ Full arm motion checked

☐ Servo torque checked

☐ Link strength considered

☐ Print orientation selected

☐ STL exported

☐ Test-fit part printed

And I strongly recommend:

Don't print the entire arm first.

We'll print small test pieces.

For example:

MG996R mounting bracket

↓

PRINT

↓

Test actual servo fit

↓

If correct → final bracket

That can save a huge amount of filament and time.

⭐ One major rule for our project

From this point onward:

No guessed dimensions for parts that physically mate with another component.

If a dimension affects:

servo ↔ bracket

or

horn ↔ link

or

screw ↔ hole

or

bearing ↔ shaft

we'll verify it.

We can use the CAD model as a starting reference, but the actual hardware gets the final word.

And your current setup is actually a great starting point

You now have:

YOUR ROBOTIC ARM

│

┌───────────┴───────────┐

│                       │

REAL HARDWARE             CAD MODELS

│                       │

MG996R Servo             MG996R Fusion

Servo Horn               Servo Horn

Electronics              Reference Arm

│                       │

└───────────┬───────────┘

│

▼

MASTER DESIGN

│

▼

3D PRINTABLE ARM

Complete Dimensions( from  google)

- Length: 40.7 mm

- Width (Thickness): 19.7 mm

- Height: 42.9 mm

- Weight: 55 g

### MG996R Servo

One important  distance :1.772

one  more  important  distance :0.2mm

- Body length = 40mm

- Body width = 19mm

- Body height = 37.50mm

Mounting-hole spacings:

- Mounting-interface  offset = 8.20mm

- mounting hole distance (1)=2mm

- mounting hole distance (2)=2.25mm

- .   Servo mounting-hole diameter =4mm

- Horn hub height =2mm

- servo output spline circle’s outer most diameter=13mm

- output  spline  length from the  servo body=5.70mm

- put spline center diameter =//

### Servo Horn

- Outer diameter = 20.00 mm

- Plate thickness = 2.00 mm

- horn   borscrew diameter=3mm

4) distance   between   two  borscrew  =7.071

### Robot Arm

- DOF = 6

- Servos = 6 × MG996R

- Link 1 length = ___

- Link 2 length = ___

- Link thickness = ___

- Joint clearances = ___

- Fasteners = ___

- Bearing arrangement = ___